| Name: | MANSI UNIYAL |
| --- | --- |
| Roll Number: | 19EE10039 |

Experiment No. 10

Name of the Experiment: Study of Differentiator and Integrator using Op amp

- Aim of the experiment

- Explain Differentiator using Op amp

- Explain Integrator using Op amp

- Tools used:

- Stimulation: Vlabs

- Connecting wires

- Resistance

- Capacitor

- Op amp

- Oscilloscope

- Voltage input

  - Square wave

  - Sinusoidal wave

- Background knowledge (brief):

- Integrator

- Output is proportional to the amplitude and time duration of the Input.

- Circuit layout is same as an inverting amplifier but the feedback resistor is replaced by a capacitor which make the circuit frequency dependent.

- WORKING: Initially (voltage is applied) the uncharged capacitor allows maximum current to pass through it and no current flows through the Op-Amp due to the presence of virtual ground, the capacitor starts to charge at the rate of RC time constant and its impedance starts to increase with time and a potential difference is develops across the capacitor resulting in charging current to decrease. This results in the ratio of capacitor's impedance and input resistance increasing causing a linearly increasing ramp output voltage that continues to increase until the capacitor becomes fully charged.

- Differentiator

- Input is connected to the 108-degree inverting output of the Op-Amp through a capacitor(C) and a negative feedback is provided to the inverting input terminal through a resistor (Rf), which is same as an integrator circuit with feedback capacitor and input resistor being replaced with each other.

- Output is the first derivative of the input signal, 180' out of phase and amplified with a factor Rf*C.

- Capacitor allows only the AC and restrict the DC.

- At low frequency the reactance of capacitor is very high causing a low gain and high frequency vice versa.

- At high frequency the circuit becomes unstable.

- Circuit (hand drawn/image)

Integrator Circuit

Differentiator Circuit

- Measurement Data (Tabular form)

- Frequency is taken as 2000 or 2500 Hz.

- Amplitude is varied from 0.5 to 1.

- Graph (Image)/Screenshots

- Integrator circuit

  - Square wave

    - Amplitude 1

    - Amplitude 0.5

  - Sinusoidal wave

    - Amplitude 1

    - Amplitude 0.5

- Differentiator circuit

  - Square wave

    - Amplitude 1

    - Amplitude 0.5

  - Sinusoidal wave

    - Amplitude 1

    - Amplitude 0.5

- Conclusion

- Integrator circuit has output where (-) sing indicates 108-degree phase shift.

- Output of differentiator is the first derivative of the input signal, 180' out of phase and amplified with a factor Rf*C.

- Capacitor allows only the AC and restrict the DC.

- Discussions

- Integration of sinusoidal function is sinusoidal

- Integration of square wave is triangle shaped function

- Differentiation of sinusoidal function is sinusoidal

- Differentiation of square wave is impulse function

- Integrator circuit layout is same as an inverting amplifier but the feedback resistor is replaced by a capacitor which make the circuit frequency dependent.