| Name: | MANSI UNIYAL |
| --- | --- |
| Roll Number: | 19EE10039 |

Experiment No. 3

Name of the Experiment:  Half Wave Rectification

- Aim of the experiment

Explain the working of a Half Wave Rectifier

- Tools used:

- Stimulation: Vlabs

- P-N junction diode

- CRO (cathode ray oscilloscope)

- Sinusoidal input voltage source

- Resistance

- Connecting Wires

- Background knowledge (brief):

  - Rectification

    - A rectifier is a device that converts alternating current (AC) to direct current (DC), a process known as rectification. Rectifiers are essentially of two types – a half wave rectifier and a full wave rectifier.

  - Half Wave Rectification

    - On the positive cycle the diode is forward biased and on the negative cycle the diode is reverse biased. By using a diode, we have converted an AC source into a pulsating DC source. In summary we have ‘rectified’ the AC signal.

    - The half-wave rectifier is a circuit that allows only part of an input signal to pass. The circuit is simply the combination of a single diode in series with a resistor, where the resistor is acting as a load.

Half Wave Rectifier

  - Half Wave Rectifiers – Waveforms

    - The output DC voltage of a half wave rectifier can be calculated with the following two ideal equations.

  - Half Wave Rectification: For Positive Half Cycle

    - Diode is forward biased, acts as a short circuit, passes the waveform through.

CASE 1

CASE 2

- Half Wave Rectification: For Negative Half Cycle

  - Diode is reverse biased, acts as an open circuit, does not pass the waveform through.

- For ideal diode

- For positive cycle

- For negative cycle

- Average output voltage

- RMS load voltage

- Average load current

- RMS load current

- Form factor

- Ripple factor

- Efficiency

- Peak inverse voltage

  - The maximum value of reverse voltage which occurs at the peak of the input cycle when the diode is reverse-biased.

- Circuit (hand drawn/image)

- Resistance (200ohms)

- Resistance (400ohms)

- Measurement Data (Tabular form)

- Resistance (200ohms)

- Resistance (400ohms)

However, both the different peak values of current and different values of series resistance we get the same value for the ripple factor (40.56 %).

- Graph (Image)/Screenshots

- Resistance (200ohms)

Sinusoidal input signal

Output rectified signal

- Resistance (400ohms)

Sinusoidal input signal

Output rectified signal

- Conclusion

    - Rectification is the conversion of alternating current to direct current.

    - Half Wave rectifier passes only half of the incoming AC power supply.

    - Value of ripple factor doesn’t depend on peak current value nor the series resistance value.

    - Ripple factor for Half Wave Rectifier is 40.56 %.

- Discussions

- Advantages of Half Wave Rectifier

  - Simple with low number of components.

  - Cheap, cost effective. However, over period of time there is more power loss, so more costly.

- Disadvantages of Half Wave Rectifier

  - They only allow a half-cycle through per sinewave, and the other half-cycle is wasted. This leads to power loss.

  - They produce a low output voltage.

  - The output current we obtain is not purely DC, and it still contains a lot of ripple (i.e. it has a high ripple factor)

- Quiz