| Name: | MANSI UNIYAL |
| --- | --- |
| Roll Number: | 19EE10039 |

Experiment No. 9

Name of the Experiment: Study of basic properties of operational amplifier:

inverting and non-inverting amplifiers

- Aim of the experiment

- Explain Inverting Op amp

- Explain Non- Inverting Op amp

- Explain Gain

- Tools used:

- Stimulation: V-labs

- Connecting wires

- Resistance

- Voltmeter

- Ammeter

- Op amp

- Background knowledge (brief):

Operational Amplifier (Op-Amp)

- It has three terminals, two high impedance input and one output terminal.

- Can perform multiple function when attached to different feedback combinations like resistive, capacitive or both.

- Used as voltage amplifier and the output voltage of the Op-Amp is the difference between the voltages at its two input terminals.

  - Open loop gain and input impedance is infinite (practically very high),

  - Output impedance and offset voltage is zero (practically very low)

  - Bandwidth is infinite (practically limited to frequency where its gain become unity).

Inverting Op-Amp

- Open loop gain of the Om-Amp is very high  makes it very unstable

- Thus, feedback is applied via external resistor (Rf) from its output to inverting input terminal (negative feedback)  reduced gain (closed loop gain, Av).

- Voltage at inverting terminal = actual input + feedback voltages

- To separate both an input resistor (Ri) is introduced in the circuit.

- Non-inverting terminal  grounded,

- Inverting terminal behaves like a virtual ground as the junction of the input and feedback signal are at the same potential.

Non-Inverting Op-Amp

- Input signal is fed to the non-inverting terminal resulting in a positive gain.

- Output voltage is in phase.

- To stabilize the circuit, negative feedback is applied through a resistor (Rf).

- Inverting terminal is grounded with an input resistor(R2).

- Circuit (hand drawn/image)

Inverting Op amp					Non-Inverting Op amp

- Measurement Data (Tabular form)

- Inverting Op amp

  - Gain: -2

    - Rf = 2, R1 = 1

  - Gain: -1

    - Rf = 2, R1 = 2

  - Gain: -0.5

    - Rf = 2, R1 = 4

- Non-Inverting Op amp

  - Gain: 2

    - Rf = 20, R1 = 10

  - Gain: 1

    - Rf = 20, R1 = 20

  - Gain: 0.5

    - Rf = 20, R1 = 40

- Graph (Image)/Screenshots

- Inverting Op amp

  - Gain: -2

    - Rf = 2, R1 = 1

  - Gain: -1

    - Rf = 2, R1 = 2

  - Gain: -0.5

    - Rf = 2, R1 = 4

- Non-Inverting Op amp

  - Gain: 2

    - Rf = 20, R1 = 10

  - Gain: 1

    - Rf = 20, R1 = 20

  - Gain: 0.5

    - Rf = 20, R1 = 40

- Conclusion

- Inverting Op amp

Thus, the gain is negative.

- Non-Inverting Op amp

Thus, the gain is positive.

- Discussions

- Gain of Inverting Op amp is negative

- Output voltage is out of phase for Inverting Op amp.

- Gain of Non-Inverting Op amp is positive

- Output voltage is in phase for Non-Inverting Op amp.