Experiment 4: Voltage-to-Frequency and Frequency-to-Voltage Conversion using LM33

Name: Mansi Uniyal

Roll no.: 19EE10039

Date: 2 February 2021

- Objective: 
Study the performance of Voltage-to-Frequency and Frequency-to-Voltage converters using LM331.

- Theory: LM331:

It is a precision voltage to frequency converter having the following important features:

- Operates on single +15 V power supply.

- Frequency range: 1 Hz to 100 kHz.

| Pin name | Pin no. | In/Out | Description |
| --- | --- | --- | --- |
| IOUT | 1 | O | Current output |
| IREF | 2 | I | Reference current |
| FOUT | 3 | O | Frequency output: open-collector output and requires a pullup resistor |
| GND | 4 | Ground | Ground |
| RC | 5 | I | R-C filter input |
| THRESH | 6 | I | Threshold input |
| COMPIN | 7 | I | Comparator input |
| VS | 8 | Supply | Supply voltage |

- Voltage-to-Frequency Converter:

Connect the circuit. The frequency output is given by:

fout=Vin2.09 V×RSRL×1RtCt                      (1)

- Simulation Assignment:

- Compute numerically the value of Vin needed to generate fout of 2.5 kHz for Rt = 6.8 kΩ, Ct = 0.01 µF, Rs = 17 kΩ and RL = 100 kΩ.

- Draw neatly the Voltage-to-Frequency converter circuit diagram in LTSpice. All the components should be chosen as ideal. Attach the screen-shot of the schematic after entering your Name and Roll No as a text on it.

- Construct the circuit, and simulate using the computed value of Vin. In the simulated waveform, measure frequency fout of the output signal Vout, and attach the screenshot of the plot.

| Vin (V) | Theoretical (kHz) | Experimental (kHz) |
| --- | --- | --- |
| 2.09 | 2.5 | 2.234 |

- Vary the value of Vin in the range of +1 V to +4 V with a step of 0.2 V, and attach the screenshot of the Vout. Plot the frequency fout versus Vin values in a curve.

with the increase in Vin, the fout increases.

|  | fout |  |  |
| --- | --- | --- | --- |
| Vin (V) | Theoretical (kHz) | Experimental (kHz) | Error (%) |
| 1 | 1.196 | 1.058 | 11.53846154 |
| 1.2 | 1.44 | 1.266 | 12.08333333 |
| 1.4 | 1.67 | 1.476 | 11.61676647 |
| 1.6 | 1.91 | 1.688 | 11.62303665 |
| 1.8 | 2.15 | 1.895 | 11.86046512 |
| 2 | 2.39 | 2.108 | 11.79916318 |
| 2.2 | 2.63 | 2.318 | 11.86311787 |
| 2.4 | 2.87 | 2.528 | 11.91637631 |
| 2.6 | 3.11 | 2.739 | 11.92926045 |
| 2.8 | 3.35 | 2.952 | 11.88059701 |
| 3 | 3.59 | 3.162 | 11.92200557 |
| 3.2 | 3.83 | 3.37 | 12.01044386 |
| 3.4 | 4.07 | 3.579 | 12.06388206 |
| 3.6 | 4.31 | 3.790 | 12.0649652 |
| 3.8 | 4.54 | 4.00 | 11.89427313 |
| 4 | 4.78 | 4.079 | 14.66527197 |

- Points to be discussed in brief:

- From the simulated fout obtained in step (d), plot the non-linearity (in %) versus Vin values (+1 V to +4 V) in a curve.

- Compute fout for the Vin range of +1 V to +4 V with a step of 0.2 V using given expression in (1). Plot the error between simulated fout (obtained in step (d)) and computed fout versus Vin values in a curve.

- Frequency-to-Voltage Converter:

Connect the circuit as shown in Fig. 4. The voltage output is given by:

Vout=fin×2.09 V×RLRS×RtCt                                 (2)

- Simulation Assignment:

- Compute numerically the value of Vout when input signal frequency fin is 2.5 kHz for Rt = 6.8 kΩ, Ct = 0.01 µF, Rs = 17 kΩ and RL = 100 kΩ.

- Draw neatly the Frequency-to-Voltage converter circuit diagram in LTSpice. All the components should be chosen as ideal. Attach the screen-shot of the schematic after entering your Name and Roll No as a text on it.

- Construct the circuit, and simulate for pulse input signal Vin with fin = 2.5 kHz, Vhigh = 15 V and Vlow = 0 V. In the simulated waveform, measure output Vout and attach the screenshot of the plot.

|  | Vout |  |
| --- | --- | --- |
| fin (kHz) | Theoretical (V) | Experimental (V) |
| 2.5 | 2.09 | 2.36 |

- Only vary the value of fin in the range of 1 kHz to 4 kHz with a step of 0.2 kHz, and attach the screenshot of the Vout. Plot the Vout versus fin values in a curve.

with the increase in fin, the Vout increases.

|  |  | Vout |  |  |
| --- | --- | --- | --- | --- |
| T (ms) | fin (kHz) | Theoretical (mV) | Experimental (mV) | Error (%) |
| 0.5 | 2 | 1672 | 1887.55 | 12.89174641 |
| 1 | 1 | 836 | 945.84 | 13.13875598 |
| 1.5 | 0.667 | 557.612 | 626.58 | 12.36845692 |
| 2 | 0.5 | 418 | 472.17 | 12.95933014 |
| 2.5 | 0.4 | 334.4 | 378.01 | 13.04126794 |

- Points to be discussed in brief:

- From the simulated Vout obtained in step (d), plot the non-linearity (in %) versus fin values (1 kHz to 4 kHz) in a curve.

- Compute Vout for the fin range of 1 kHz to 4 kHz with a step of 0.2 kHz using given expression in (2). Plot the error between simulated Vout (obtained in step (d)) and computed Vout versus fin values in a curve.